NLAS

ANNUAL REPORT 2022

Author:

Jihad Bachir

Mahmoud Al Zohbi

Raja Murad

Last Update:

27.01.2023 09:47

Table of Contents

Preface	1

Introduction	2

Project 1: Electric TukTuk Testing Rig	3

1.1.	Requirments	3

1.2.	Mechanical design	9

1.3	echanical Realization	11

1.4	System test:	31

Project 2: Electric TukTuk Testing 1 Rig	36

1)	Requirements	36

1-	Agriculture Electric Real Size Tractors Products in market:	36

2-	Agriculture Machines:	39

2	Mechanical design:	48

3	Mechanical Realization:	53

2)	Work Plan Update :	55

1	Taking apart all the Front suspension mechanism:	58

2	Brake oil tank filling	61

3	Changing the Rear Tires with dirt agriculture tires:	62

4	Lifting the Cabin 4 cm:	63

5	Solar Panels:	64

6	Solar Panels Stand:	64

7	Stand painting:	70

8	Front wheel mechanism:	73

9	Hitch assemble	74

Project 3: Electric TukTuk 2	82

1	Requirments and planning:	82

2	design a new agriculture machine :	84

3	Mechanical Realization:	84

4	Assembling the TUKTUK:	86

5	Lifting the cabin 4 cm:	87

6	Solar panels stand installation:	87

7	Agriculture water sprayer machine:	92

8	1500 w motor and wires:	94

References	98

# Preface

This report contains details of the NLAS Institute project that we carried out for the year 2022.

The Project presented is the E-TRACTUK

An electric tractor based on a TukTuk Chassis.

# Introduction

Solar Cell Tuk Tuk have clean driving power, without any pollution, saving environment, for comfortable transportation and with more features for your convenience. Wider space for passengers than general Tuk Tuk.Easy for maintenance, just charge the battery and check the battery acid level.Turning controlled by steering, make it easier in controlling than the conventional Tuk Tuk. More space for driver, without engine, heat and noise.

# Project 1: Electric TukTuk Testing Rig

## Requirments

Work on this project has started in 17 October 2022

- This is a list of system requirments

| Wheels nb | 3 wheels |
| --- | --- |
| Speed | 70 km/h max |
| Climbing Ability | 20 - 30 degree |
| Nb of passangers | 2 |
| Max Load | 350 kg (changable ) |
| Budget | 1500$ |
| Nb of operation Hr | 6 |
| Solar rechargeable | yes |
| Reverse gear | yes |
| Water resist | yes |
| Operation Temp | -10 to +50 degree Celsius |
| Lifetime | 3 years/100.000 km |
| pedal type or throtle | pedal |
| Material | Metal |
| Charging hours | 6 h |
| Breaking | Front and Rear oil drum brake |
| Lighting | LED |
| Seats material | Leather |

- Power/drive Train and parts::

https://skill-lync.com/student-projects/electric-rickshaw-modelling-2

- Similar products

Al halabi E-BIKE Tripoli, Koura square

مثل هيك تكتك لنقل البضاعة و يمكن ان يعدل لنقل الركاب
بسعر ١٥٠٠-١٦٠٠$
سرعة ٦٠ كم/س
يمشي مسافة ٤٠-٥٠ كم
تشريع ٥ ل٨ ساعات
محرك ١٠٠٠ وات
٤ بطاريات
يمكنه ان يصعد طلعة الهيكلية لان لديه GearBox من فراش واحد

الحلبي لديه كل ما يتعلق بالتكتك من محركات لميكانيك للcontrollers ...

عرض اسعار القطع

- SmarTuk

https://www.tuvie.com/smartuk-modular-electric-tuktuk-design-for-cambodia/

Designed by Vincent Chan and Andy Lee, SmarTuk features modern design with better functionality that help to create better image on the street. No more dirty vehicles moving around the street, polluting environment.

Here are some advantages of SmarTuk design:

- Modular loading bay for selling different types of goods

- Improved water resistance as flood is common in Cambodia

- Customer can order or locate specific types of goods anytime

- GPS radio so the vehicle could be tracked real time

- Security box features

- AM Electric Bike (lebanon)

https://amebike.business.site/?utm_source=gmb&utm_medium=referral

- - Solar Panels:

https://www.alibaba.com/product-detail/Bluesun-ETFE-solar-flexible-160w-170w_1600281712498.html?

  - Mechanical design

In this paragraph we will present the mechanical design of TukTuk chassis

  - Mechanical Realization

Assembling the TukTuk

TukTuk Chassis

1000w controller

1000w controller and connecting wires

Throtle handelbar and speed controller

Brakes wires

5 12v batteries and charger

Rear suspension, shock absorber, and brake drum

GearBox shifter

  - System test:

- Test 20-10-22:

All wires were connected to the controller.

The tuktuk starts to go forward but the dashboard, lighting and flashers were not working.

The main power cable of the dashboard, lighting and flashers was not connected to the 60v batteries, after connecting it, everything starts to work normally but the TukTuk went just backward.

The Flashers coil failed.

Throtle plug

- Test 26-10-22:

Recheck for all the Throttle handlebar and Forward/Reverse wires.

The TukTuk starts to work normally forward and reverse.

The Forward/Reverse wires were connected incorrectly and switched with the flasher wires.

Videos on CD:

Testing Forward/Reverse switch video

Testing the three speed levels videos(Low,Medium,High)

- Test 27-10-22:

After changing the flashers coil and the flashers were working well.

the E-TukTuk was tested with load in the inclined round of Haykal emergency entrance:

- in low/medium and high speed, the tuktuk climbed the inclined round but slowly and difficultly

- using the gear box it climbed the slop easily but also slowly

2 Videos on CD

After assembling all the cabin the tuktuk is tested with the load of the cabin and 4 persons in back and the driver and the tuktuk goes smoothly without facing any problem

# Project 2: Electric TukTuk Testing 1 Rig

Requirements

- Agriculture Electric Real Size Tractors Products in market:

1-https://www.pinterest.com/pin/284641638921221021/

2- https://www.motherearthnews.com/sustainable-living/green-transportation/our-solar-powered-tractor-zbcz1307/

3- https://solarimpulse.com/solutions-explorer/solectrac-electric-tractors

Mini agriculture TukTuk products:

1- https://www.indiamart.com/proddetail/shakti-three-wheels-mini-tractor-16757725962.html

2- https://kishankart.com/product/mini-tractor-tractbull/

- Agriculture Machines:

- Wheel chains for dirt roads can be used:

- Front and rear wheels can be replaced by this type and size of tires but the rear ones should fit the wheel cover space and drum width

-Rear wheels from Jamal And Chaaban for 90$:

- Simplest Hitch design:

- Solar panels used:

- Overview, Requirements lists:

Project Planning :

Project presentation:

Plan

- Supplier for Tractor parts in tripoli:

- Mechanical design:

FreeCAD design of the agriculture tuktuk

17-22-2022 drawing:

28-11-22 E_TukTuk with solar panel stand:

30-11-22 E_TukTuk with solar panel stand and hitch handle:

5-12-22 E_TukTuk:

17-12-22 :

19-12-22 E_TukTuk with solar panel stand:

| E_TukTuk with solar panel stand: |  |
| --- | --- |

| Hitch Design |  |
| --- | --- |

Front wheel rim disc design:

This disk is designed to fit the agriculture front rim

- Mechanical Realization:

1 Testing stand:

Work Plan Update :

2-1-2023

29-12-2022

28-12-2022

13-12-2022

8-12-2022

26-11-2022

  - Taking apart all the Front suspension mechanism:

  - Brake oil tank filling

  - Changing the Rear Tires with dirt agriculture tires:

  - Lifting the Cabin 4 cm:

  - Solar Panels:

  - Solar Panels Stand:

The stand is adjustable and can be modified

it can hold up to 5 solar panels

  - Stand painting:

  - Front wheel mechanism:

The goal is to install a large suspension to use the same large dirt tire used in the rear suspension,

we design a new drum and rim:

  - Hitch assemble

# Project 3: Electric TukTuk 2

- Requirments and planning:

14-1-2023:

12-1-2023

COST estimated: 2000$

- tuktuk chassis with parts: 1350$

(aL HALABI E-BIKE tripoli koura square 76905031)

with 1500w motor and controller

- Steel for solar panels stand and hitch : 80$

(SHENDER STEEL Tripoli Al Zahiriye 03230994 )

- 4 Solar panels: 148$

(SMART SECURITY Tripoli bahsas 71061010 )

- MPPT : 150$

(AL HALABI Tripoli zehriye bolevare )

- Front suspension with rim and rear tires : 150$

(AL HALABI Tripoli zehriye bolevare )

Rear tires (JAMAL AND SHAABAN Tripoli 200 square 06424752)

- welder`s work : 80$

(Ahmad Bizre 76525575 )

- hitch lever: 20$

...

- design a new agriculture machine :

Water Sprayer

- Mechanical Realization:

Buying the tuktuk`s parts without:

-The front suspension

-Rear tires

- Assembling the TUKTUK:

- Lifting the cabin 4 cm:

- Solar panels stand installation:

- Agriculture water sprayer machine:

- 1500 w motor and wires:

# References